## Category L Type Approval requirements

From 1 January 2025, a number of changes for the first registration or entry into service of EU Category L vehicles comes into force under EU Regulations 168/2013 and 134/2014.

This document provides an overview of the regulatory changes and the eligibility criteria for vehicles within EoS applications.

The RSA is granting end-of-series derogations to eligible vehicles that don’t meet the updated requirements outlined below.

| Derogation Eligibility Requirements* |  |
| --- | --- |
| Primary Eligibility Criteria | Valid CoC issued on or before 31/12/2024 |
| Derogation Limits | 100 vehicles or 10% of vehicles first registered in Ireland in the preceding 2 years** whichever is higher |

##### * Additional eligibility criteria will apply depending on the requested derogation

##### ** Evidence of registered vehicles may be requested

Note: Where a vehicle is granted a derogation from one of the below regulations, further end of series applications for that vehicle will not be processed.

## Noise Emissions

Regulation (EU) No. 134/2014 – outlines the type-approval requirements for category L vehicles regarding their environmental and propulsion unit performance.

From 1 January 2025, EU category L vehicles are required to comply with the noise emissions requirements outlined in the table below.

| 1 January 2025, Noise Emissions Requirements |  |
| --- | --- |
| Vehicle Category | UNECE Regulation No. |
| L1e | 63.02 |
| L2e, L4e, L5e, L6e, and L7e | 9.07 |
| L3e | 41.05 |

##### Additional Noise Emissions Eligibility Criteria:

- Category L3e and L4e noise emissions must be type approved to ECE 41.04

## Motorcycle OBD Requirements

Regulation (EU) No. 168/2013 - on the approval and market surveillance of two- or three-wheel vehicles and quadricycles.

From 1 January 2025, certain EU category L vehicles are required to comply with the Euro 5 OBD requirements as outlined in the table below.

| 1 January 2025 OBD requirements |  |  |
| --- | --- | --- |
| OBD Requirements | Description | Scope |
| Stage I | Requires vehicles to be fitted with an onboard diagnostic (OBD) system meeting the OBD Stage I requirements including any operating mode which significantly reduces engine torque using the Euro 5 OBD threshold limits. | Applies to: L3e, L4e, L5e, and L7e |
| Stage II | Requires vehicles to be fitted with an onboard diagnostic (OBD) system meeting the OBD Stage II requirements, including those relating to catalytic converter monitoring, using the Euro 5 OBD threshold limits. | Applies to: L3e (except L3e-AxE and L3e-AxT), L4e, L5e-A, and L7e-A |

##### Additional OBD Eligibility Criteria:

- Type approved on or before 31 December 2023

- Type approved to Euro 5 emissions